Memorandum

To: Carolina Beach Town Manager

From: JANT Engineering

Date: September 13, 2022

Subject: Environmental Assessment Initial Draft

____________________________________________________

Purpose

The purpose of this memorandum is to supply an environmental assessment of a proposed area of shoreline found in Carolina Beach, North Carolina, where a storm reduction designed project is to be proposed. This assessment includes analyses of site characteristics such as topography/bathymetry, existing habitats and species, land use, shoreline trends, and storm characteristics/meteorology.

Topography and Bathymetry

Survey data from 2014-2021 was provided by the U.S. Army Corps of Engineers at the proposed project site in Carolina Beach. The data includes surveys along 23 transects for each of the aforementioned years. This data was analyzed to understand spatial bounds, bathymetry, and volume change over the provided time period. Survey locations in geo-spatial points for each year can be seen in Figure 1. The red star indicates the transect that was used from year-to-year comparisons.

Figure 1. Transect locations in Carolina Beach, NC from 2014-2021. The transects from year-to-year are relatively close.

With rather uniformed spacing of raw data, a linear interpolation was used to create a grid for the data points. The grid contains a cross-shore resolution of 1 ft and an alongshore resolution of 100 ft. The grid with the selected transect can be seen in Figure 2. The transect comparisons can be seen in Figure 3.

Figure 2. Grid (blue) with the transect used for year-to-year comparisons (red).

Figure 3. Transect comparison from 2014-2021.

There were significant increases in the berm and dune elevation (~3ft and 2ft, respectively) in 2019 due to the beach nourishment project that followed Hurricane Florence in 2018 (Figure 3). In 2020, the berm was eroded by up to 5ft in many areas along the coastline, relative to the year prior. A comparison in elevation from 2019 to 2020 and from 2020 to 2021 can be seen in the digital elevation model (DEM) in Figure 4. The northern end of the beach experienced the most erosion between 2019 and 2020 (Figure 4A.), as well as between 2020 and 2021 (Figure 4B.). The changes in elevation indicate that significant beach erosion occurred at the site, especially in the northern part of Carolina Beach.

Figure 4. (A.) Elevation difference from 2019 to 2020. (B.) Elevation difference from 2020 to 2021.

Carolina Beach was also renourished in early 2022 due to significant erosion and flooding of surrounding areas. An additional dataset will be collected on 19 September 2022 to add to the current data for further use in understanding erosion and accretion patterns at the site.

Existing Habitats, Species, and Wildlife
It is imperative to consider the habitats and native species existing in the proposed project area to minimize any potential disruption to the local environment. Including, but not limited to, sea turtles (Loggerhead, Kemp’s Ridley, Leatherback), American Alligators, beach nesting birds (Least Tern, Wilson’s Plover, Black skimmer), and American Beach Grass. “The nesting season for Loggerhead Sea Turtles begins in April and ends in September, with the peak in June. They emerge onto the beach at night every 14 days, laying an average of four clutches containing roughly 100 to 120 eggs in each” (Sea Turtle). Wildlife Commission states that “signs help people avoid nesting grounds from April 1 through Aug. 31 — the sensitive nesting and brood-rearing season — and advise that entering an area can result in the loss of eggs or chicks” (Wharton, M). The American Alligator, as noted from the Smithsonian’s National Zoo & Conservation Biology Institute, nest around late June and early July, and the female typically lays 35 to 50 eggs, while some can lay up to 90 eggs. The eggs are then covered with vegetation and hatch after a 65-day incubation period” (American alligator). American beach grass as stated by Hoffman Nursery, “This grass helps build dunes by weaving intricate rhizomes, which can extend more than three feet beneath the sand and expand rapidly. American Beach Grass depends on wind to whip up the sand around its stems.” Depending upon the time of year, certain species migrate to the backshore for nesting during mating season and create areas unsuitable for construction. See Appendix A for reference photographs of species.

Land Use

Based on 2014 land use data publicly provided by New Hanover County, properties along the Carolina Beach coastline can generally be classified into one of these three categories: residential, services, and land and water area (Figure 5). Approximately half of the coastline is classified as land and water area, while the remaining half is split between residential and service properties.

Figure 5. A map of existing land use in New Hanover County, 2014.

Shoreline Trends

Carolina Beach has an East-Southeast (ESE) facing shoreline with predominantly southward longshore sediment transport. Based on available data, the shoreline in the proposed project area eroded by an estimated 60-90 meters (200-300 feet) from 1857 to 1933 (Figure 6). It is important to note that beach nourishment projects have taken place in Carolina Beach since 1955 and have had direct impacts on the location of the shoreline (Table 1). The general trend that can be seen from the shoreline data in conjunction with the prior nourishment project information is that Carolina Beach is eroding and is sustained by the beach nourishment projects that now occur cyclically every three years. The North Carolina 2019 Oceanfront Setback Factors & Long-Term Average Annual Erosion Rate Update Study: Methods Report states that accretion was measured along 2.5 miles (65.1 percent) of Carolina Beach, while erosion was measured along the remaining 1.3 miles (34.1 percent). Additionally, the report states that the average blocked erosion rate at Carolina Beach is 2.5 feet per year.

Figure 6. A map showing the changing shoreline in Carolina Beach from years 1849 to 2020.

Table 1. Beach nourishment projects completed in Carolina Beach, NC from 1955-2016. Source: Beach Nourishment Database compiled and supported by the Program for the Study of Developed Shorelines (PSDS), Western Carolina University.

|  | Location | Year Completed | Primary Funding Source | Justification | Length (ft) | Volume (CY) | Nominal Cost | 2021 Real Cost |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Carolina Beach | 1955 | Federal | Shore Protection | 0 | 252,000 | $50,000 | $505,541 |
| 2 | Carolina Beach | 1956 | State | Unknown | 0 | 200,000 | $0 | $0 |
| 3 | Carolina Beach | 1965 | Federal | Shore Protection | 13,992 | 2,632,000 | $1,025,000 | $8,817,278 |
| 4 | Carolina Beach | 1967 | Federal | Emergency | 4,224 | 389,959 | $186,308 | $1,511,493 |
| 5 | Carolina Beach | 1968 | Federal | Navigation | 0 | 97,000 | $0 | $0 |
| 6 | Carolina Beach | 1970 | Federal | Emergency | 4,224 | 346,000 | $291,159 | $2,033,385 |
| 7 | Carolina Beach | 1971 | Federal | Shore Protection | 11,600 | 734,140 | $517,897 | $3,465,051 |
| 8 | Carolina Beach | 1972 | Federal | Navigation | 182 | 18,816 | $0 | $0 |
| 9 | Carolina Beach | 1973 | Federal | Navigation | 0 | 30,547 | $0 | $0 |
| 10 | Carolina Beach | 1974 | Federal | Navigation | 0 | 66,687 | $0 | $0 |
| 11 | Carolina Beach | 1975 | Federal | Navigation | 0 | 40,804 | $0 | $0 |
| 12 | Carolina Beach | 1976 | Federal | Navigation | 0 | 119,971 | $0 | $0 |
| 13 | Carolina Beach | 1977 | Federal | Navigation | 0 | 62,066 | $0 | $0 |
| 14 | Carolina Beach | 1979 | Federal | Navigation | 0 | 230,866 | $0 | $0 |
| 15 | Carolina Beach | 1981 | Federal | Emergency | 0 | 406,352 | $1,051,774 | $3,135,305 |
| 16 | Carolina Beach | 1981 | Federal | Navigation | 0 | 109,176 | $174,002 | $518,694 |
| 17 | Carolina Beach | 1982 | Federal | Shore Protection | 14,256 | 3,662,181 | $8,384,406 | $23,543,238 |
| 18 | Carolina Beach | 1983 | Federal | Navigation | 0 | 119,244 | $0 | $0 |
| 19 | Carolina Beach | 1985 | Federal | Shore Protection | 6,000 | 764,162 | $1,652,004 | $4,160,256 |
| 20 | Carolina Beach | 1985 | Federal | Navigation | 0 | 28,267 | $0 | $0 |
| 21 | Carolina Beach | 1988 | Federal | Shore Protection | 5,700 | 950,913 | $1,890,535 | $4,330,332 |
| 22 | Carolina Beach | 1989 | Federal | Navigation | 0 | 98,843 | $0 | $0 |
| 23 | Carolina Beach | 1991 | Federal | Shore Protection | 11,600 | 1,008,736 | $2,450,286 | $4,874,846 |
| 24 | Carolina Beach | 1995 | Federal | Shore Protection | 11,600 | 1,157,742 | $3,185,642 | $5,664,130 |
| 25 | Carolina Beach | 1998 | Federal | Shore Protection | 0 | 575,000 | $2,020,650 | $3,359,114 |
| 26 | Carolina Beach | 2001 | Federal | Shore Protection | 0 | 1,204,646 | $2,964,355 | $4,535,580 |
| 27 | Carolina Beach | 2004 | Federal | Shore Protection | 0 | 680,000 | $2,909,500 | $4,173,569 |
| 28 | Carolina Beach | 2007 | Federal | Shore Protection | 5,280 | 532,250 | $3,000,000 | $3,921,418 |
| 29 | Carolina Beach | 2010 | Federal | Shore Protection | 0 | 690,000 | $4,278,185 | $5,315,267 |
| 30 | Carolina Beach | 2013 | Federal | Shore Protection | 14,256 | 900,000 | $5,291,166 | $6,153,422 |
| 31 | Carolina Beach | 2016 | Federal | Shore Protection | 3,328 | 890,000 | $5,000,000 | $5,645,208 |

Meteorology and Storm Characteristics

Meteorological and storm characteristics consist of tides, wind speed and direction, air temperature, and barometric pressure. Table 2 is the range of wind, air temperature, and barometric pressure at Carolina Beach during four sections of the year generally aligned with each season.

Table 2. Meteorological observation ranges for wind, air temperature, and barometric pressure at Carolina Beach.

Wind (knots)

Air Temperature (F)

Barometric Pressure (mb)

Jan-March

1 -- 30

29 -- 68

1000 -- 1035

Apr-Jun

2 -- 25

38 -- 90

1001 -- 1032

Jul-Sept

2 -- 37

61 -- 90

1004 -- 1026

Oct-Dec

2 -- 22

37 -- 82

999 -- 1031

Meteorological Observation Ranges at Carolina Beach

**wind values are constant, NOT gusts**

Carolina Beach experiences a semi-diurnal tide with tide heights varying depending on the time of year and current weather conditions. According to 2019 USACE Carolina Beach Renourishment Evaluation Report, Carolina Beach experienced 21 hurricanes and 29 tropical storms within a 50-mile radius prior to 1964. Since 2014, there have been seven hurricanes to impact Carolina Beach including Arthur (2014), Hermine (2016), Matthew (2016), Florence (2018), Dorian (2019), Fay (2020), and Isaias (2020).

Conclusion

For JANT Engineering, determining the most beneficial and minimally disruptive storm reduction project is our primary goal. Our main design considerations, based on this environmental assessment, include topographic and bathymetric features, existing habitats and wildlife, shoreline trends, meteorology, and storm characteristics. Two species of consideration are sea turtles and nesting birds, as these animals generally tend to nest on beaches from April to October. If possible, project construction should take place outside of this time frame. The remaining topics of this assessment (i.e., topography and bathymetry, land use, shoreline trends, meteorology, and storm characteristics) will factor into design sea state conditions for functionality and structural testing.

Appendix A

Loggerhead Sea-Turtle

American Alligator

Least Tern

Wilsons Plover

Black Skimmer

American Beach Grass

Works Cited

Cornell Lab of Ornithology. (n.d.). Wilson's plover identification, all about birds. All About Birds. Retrieved September 12, 2022, from https://www.allaboutbirds.org/guide/Wilsons_Plover/id

American Alligator. National Wildlife Federation. (n.d.). Retrieved September 12, 2022, from https://www.nwf.org/Educational-Resources/Wildlife-Guide/Reptiles/American-Alligator

American alligator. Smithsonian's National Zoo. (2019, September 20). Retrieved September 12, 2022, from https://nationalzoo.si.edu/animals/american-alligator#:~:text=The%20nest%20can%20measure%20seven,a%2065%2Dday%20incubation%20period.

American Beach Grass. Hoffman Nursery. (n.d.). Retrieved September 12, 2022, from https://hoffmannursery.com/plants/details/ammophila-breviligulata

ArcGIS web application. (n.d.). Retrieved September 12, 2022, from https://nhcgov.maps.arcgis.com/apps/webappviewer/index.html?id=bf12527f385b4d2e8420d892863b31dd

“Beach Nourishment - Carolina Beach, NC.” Beachnourishment.wcu.edu, Program for the Study of Developed Program, WCU, https://beachnourishment.wcu.edu/test?state=NC&beach=Carolina+Beach.

Black Skimmer. Audubon. (2022, April 1). Retrieved September 12, 2022, from https://www.audubon.org/field-guide/bird/black-skimmer#photo5

Broome, S. (n.d.). Restoration and management of coastal dune vegetation: NC State Extension Publications. Restoration and Management of Coastal Dune Vegetation | NC State Extension Publications. Retrieved September 12, 2022, from https://content.ces.ncsu.edu/restoration-and-management-of-coastal-dune-vegetation

Carolina Beach, NC Beach Renourishment Evaluation Report. (n.d.). Retrieved September 12, 2022, from https://www.saw.usace.army.mil/Portals/59/docs/coastal_storm_damage_reduction/Carolina%20Beach/Carolina%20Beach%20%20-%20DRAFT%20Beach%20Renourishment%20Evaluation%20Report%20-%20USACE%20-%20June%2024%202019.pdf

Joseph F. van Gaalen, Sarah F. Tebbens, and Christopher C. Barton. Longshore Sediment Transport Directions and Rates from Northern Maine to Tampa Bay, Florida: Literature Compilation and Interpretation. Journal of Coastal Research 32(6), 1277-1301, (1 November 2016). https://doi.org/10.2112/JCOASTRES-D-15-00002.1

Least tern identification, all about birds, Cornell Lab of Ornithology. All About Birds, Cornell Lab of Ornithology. (n.d.). Retrieved September 12, 2022, from https://www.allaboutbirds.org/guide/Least_Tern/id

“North Carolina 2019 Oceanfront Setback Factors & Long-Term Average Annual Erosion Rate Update Study: Methods Report.” North Carolina Division of Coastal Management, North Carolina Division of Coastal Management, 16 Jan. 2019, https://files.nc.gov/ncdeq/Coastal%20Management/GIS/2019_Oceanfront_Setbacks_ErosionRate_Update_Report.pdf.

Sea Turtle - North Carolina Wildlife Resources Commission. (n.d.). Retrieved September 12, 2022, from https://www.ncwildlife.org/portals/0/Conserving/documents/FactSheets/nongame_seaturtle_lores.pdf

Wharton, M. (2022, June 8). Wildlife Commission asks beachgoers to watch for nesting birds. N.C. Wildlife Resources Commission. Retrieved September 12, 2022, from https://www.ncwildlife.org/Connect-With-Us/wildlife-commission-asks-beachgoers-to-watch-for-nesting-birds-2